Creating physiology regressors using the PhysIO toolbox

October 2023; additional info and further read: https://www.sciencedirect.com/science/article/pii/S016502701630259X?via%3Dihub

Required:

- Json- files of functional data for each run

- Physiology log-files, one for all runs (.resp and .puls files)

Figure 1. Puls and resp-file.

- Script: /storage/nv_shared/PhysIO_excitex/PhysIO_get_regressors.m

Before running the script:

- Always check if on path:

SPM: /usr/local/spm12/

Toolbox-code modified for our scanner model: /storage/nv_shared/software/PhysIO/tapas-master/PhysIO/code

- Individual parameter settings in the script:

- physio.log_files.vendor = 'Siemens_xa20'; % specific for our scanner model.

- physio.log_files.resp_type = 'spine'; % set to spine or belt, depending on type of physiology correction.

- physio.model.orthogonalise = 'RETROICOR'; % select depending on physiology model used, e.g. 'RETROICOR', or c, r; to avoid correlations between regressors.

- physio.model.retroicor.include = true; % select physiology model here: RETROICOR, RVT or HVT.

physio.model.retroicor.order.c = 3;

physio.model.retroicor.order.r = 4;

physio.model.retroicor.order.cr = 1;

- % select orders for RETROICOR model; here the default option based on Glover et al., 2000 is set.

- physio.model.rvt.include = false; % select physiology model here.

- physio.model.hrv.include = false; % select physiology model here.

- physio.model.other.include = false; % select physiology model here.

- physio.verbose.level = 3; % 2 or 3, will produce control plots – useful to check quality of data and  if synchronization has worked; when analyzing several participants it’s best to set to 0.

After running the script:

- Validation of results  Check the diagnostic plots the toolbox delivers:

- Check the raw time series for artifacts/ detachments (4A)

- Check distribution of raw physiological data (should be left-skewed, 4B), are there any indications for ceiling effects or temporary detachment (2)?

- Check period used for synchronization, does it fit? (4C)

- Check filtered time series (are there still severe artifacts?)

- Check regressor file (3): regressors should have a value for each slice and 18 regressors if you specified RETROICOR (i.e., 6 cardiac phase regressors, 8 respiratory phase regressors, and 4 interaction terms).

- Regressors can then be added to a GLM.

Figure 2: Artifacts in raw respiratory signal due to issues during recording with the breathing belt.

Figure 3. Final output (RETROICOR regressors)

Figure 4: Steps from raw data to the final regressors.